# SPECIAL PROVISIONS

# PROJECT TITLE (LIMITS)

# PROJECT NO.

This stamp certifies the attached modifications to the ISPWC and ACHD supplemental provisions.  It does not certify the ISPWC or ACHD supplemental provisions.

# GENERAL NOTES:

- ACHD OWNED CONDUIT IMPACTS

ACHD-owned conduit may be shallow and located directly behind the curb or under the sidewalk.  The Contractor’s method of sidewalk and/or curb removal shall be done in a manner to avoid damage to conduit and fiber.  Any damage shall be repaired at the Contractor’s expense.

- AMERICANS WITH DISABILITY ACCESS (ADA) DURING CONSTRUCTION

The needs and control of all road users (motorists, bicyclists, and pedestrians within the highway and/or public right-of-way, including persons with disabilities in accordance with the Americans with Disabilities Act of 1990 (ADA), Title II, Paragraph 35.130) through a temporary traffic control “TTC” zone shall be an essential part of highway construction, utility work, maintenance operations, and the management of traffic incidents.  The primary function of TTC is to provide for the reasonably safe and efficient movement of road users through or around TTC zones while reasonably protecting workers, responders to traffic incidents, and equipment.

Temporary facilities, including reasonably safe pedestrian routes around work sites, are also covered by the accessibility requirements of the Americans with Disabilities Act of 1990 (ADA) (Public Law 101-336, 104 Stat.327, July 26, 1990. 42 USC 12101-12213 (as amended)).  Implementation of TTC plans and installation and maintenance of devices shall be the responsibility of the contractor performing the construction, alteration and/or maintenance of the highway or public right-of-way.  Temporary traffic control for pedestrians shall meet the accessibility requirements (Standards) set forth in Part 6 of the MUTCD.  When an existing continuous sidewalk or street crossing route cannot be maintained for pedestrians because of construction, either temporary walkways with curb ramps are to be provided, or the construction shall be phased to maintain access to the affected addresses.  Contractors shall be allowed flexibility as long as the requirements are met.

The location of the construction project and whether or not accessible facilities are present shall also determine the extent of the needed temporary facilities. The contractor is only required to maintain practical continuity where accessible facilities already exist. On low speed rural roads that do not have sidewalks and are used by bicyclists, no additional measures are needed as the bicycles can share the available travel lanes with other traffic.  On moderate to higher speed rural roads, if a bike lane exists then it should be properly detoured, complete with signage, to provide a safe route through or around the work area.  If a road or bridge project affects vehicular traffic to a business, residence, school or any other type of pedestrian generating location with existing accessible facilities, then pedestrian and handicapped access must be maintained.

A continuous route for all pedestrians, including the disabled and bicyclists, shall be maintained at all times.  When existing pedestrian facilities are disrupted, closed, or relocated in a TTC zone, the temporary facilities shall be detectable and include accessibility features consistent with the features present in the existing pedestrian facility.  The temporary route should enable pedestrians to bypass the construction site while minimizing the retracing of their steps or going significantly out of their way.  Additional consideration must be given to the disabled since they may not have the physical or cognitive ability to improvise (e.g. balancing along the curb or a very narrow path) or use unofficial alternatives (e.g. using an adjacent grass surface).  Temporary routes must meet the accessibility guidelines of the ADA for permanent facilities and shall be marked with the proper signage. Should existing crosswalks at signalized intersections be closed or made inaccessible, temporary crosswalks should be painted in an accessible location.  Temporary signals should include pedestrian phases.

Contractors shall not block temporary walkways with contractor parking, materials piles, signs, rubble or rubbish.  Construction equipment and equipment operation must be separated from the temporary walkways.  At work zones where higher volumes of pedestrian traffic or school children exist, pedestrian fences or other protective barriers may be needed to prevent access into the construction area.

Detour and diversion routes, when used for pedestrians and bicyclists, should be evaluated for the following items:

- Direct conflicts between pedestrians and vehicular traffic, work vehicles, and other work activities must be reduced with protective barriers or continuous high contrast fencing (min 36” high with a 6” high toe board).  See MUTCD 6F.68 and 6D.02

- Temporary pedestrian facilities should provide safe, accessible routes that replicate as nearly as practical the most desirable characteristics of the existing facility, and parallel the disrupted route whenever possible.  A smooth, continuous hard surface should be provided throughout the entire length of the temporary pedestrian facility.  There should be no curbs or abrupt changes in grade or terrain that could cause tripping or be a barrier to wheelchair use.  The geometry and alignment of the facility should meet the applicable requirements of the Americans with Disabilities Act Accessibility Guidelines (ADAAG) for Buildings and Facilities.  See MUTCD 6D.01 and 6D.02

- Advance information placed at appropriate distances before the work zone allowing pedestrians to make timely decisions about routes through or around the work zone.  See MUTCD 6F.14

- Transition information allowing pedestrians to find a safe path through and around work zones, which is critical when the pathway is restricted, diverted or detoured.  See MUTCD 6F.14

- Work area information assisting in safe passage of pedestrians through the work zone.  This information is needed on all pedestrian routes except detours.  See MUTCD 6F.14

- Exit information directing pedestrians back to the original route.  See MUTCD 6F.14

- Crosswalk placement at intersections may need additional signage, temporary striping, traffic signal modification, pedestrian signals with audible alarms if justified, proper push button height, and ramps.  See MUTCD 6H.29, 6F.80, and 4E.06

- Accommodations for other transit forms (busses, trains etc.) are made.  See MUTCD 6D.02

- Requirements of the ADAAG and MUTCD are adhered to.

- Access is maintained to the affected businesses and residences.

- ASPHALT AND CONCRETE CUTTING

Asphalt pavement and concrete cutting shall be incidental to other items of this contract.  Contractor shall ensure that cuts made in asphalt and concrete for trenches are parallel and do not wander or diverge from the intended line.

Dust produced by asphalt or concrete saw cuts shall be collected using a high-powered vacuum dust control system, eliminating the dust from entering the atmosphere.  The suction device shall be attached to the cutting equipment or positioned to a assure a maximum amount of dust will be collected before it can be released into the atmosphere.  The cost to ensure dust is collected during asphalt and concrete cutting shall be considered incidental to the cost of the project and no additional compensation will be made.

- BASIS OF PAYMENT

Except as modified herein, the various roadway work called for on the Bid Schedule shall be performed, measured, and paid for as indicated on said Bid Schedules and as provided in the 2017 IDAHO STANDARDS FOR PUBLIC WORKS CONSTRUCTION (ISPWC) and the latest supplements. The Contractor is required to have the current edition of the ISPWC standards to perform this work. Unless otherwise specified herein, all plan quantities are based upon in-place, completed and accepted units.

- BOISE CITY PARKING METER IMPACTS

Before removal of asphalt on any downtown streets with Boise City Parking Meters, the Contractor shall notify Boise City Parking in writing at least 15 days prior to the start of work to remove their parking sensors embedded in the asphalt.  The Contractor shall be responsible for replacement costs of the sensors if this requirement is not met.  Contact Boise City Parking Services, Shane Anderson, 208-972-8183, sanderson@cityofboise.org.

- CONFLICTING PAVEMENT MARKINGS

All conflicting pavement markings shall be completely obliterated using an ACHD approved method prior to applying permanent pavement markings.  The cost of performing all obliteration shall be considered incidental to other items and no separate pay item will be included for obliteration of pavement markings.

- CONNECTING PIPES

Connecting new pipes, manholes, irrigation structures\ditches and catch basins to existing pipes, manholes, irrigation structures\ditches and catch basins is incidental to other work and no additional compensation will be made.

- CONSTRUCTION STAGING

The Contractor shall furnish ACHD with a detailed construction staging plan based on their anticipated approach to construction activities. This plan and implementation will be the responsibility of the contractor. Final approval of the staging plans and revisions thereof must be granted by ACHD prior to implementation.

Staging plans that may be contained within the project plans are primarily for bidding purposes only. The contractor may utilize any staging plans contained within the bid documents as a basis for staging but the final staging plan will be provided by the contractor prior to construction activities.

The cost associated with providing the necessary staging plans will be considered incidental to the project and no additional compensation will be made.

- DAMAGED AREAS BEYOND SPECIFIED CONSTRUCTION LIMITS

Damage to any areas or items not specifically identified for removal, including the existing pavement, sod, sprinklers and landscaping in the contract documents shall be promptly repaired by the contractor.  The cost to complete such repairs shall be considered incidental to the cost of the project and no additional compensation will be made.

- EXCESS SITE MATERIAL

The Contractor shall be responsible for providing a site for the disposal of excess or unsuitable materials.  If bituminous material is to be disposed of, the site shall meet the requirements of the Idaho Department of Environmental Quality.  All excess material sites shall be approved by ACHD.  No separate payment will be made for the acquisition or operation of the sites, or for loading, hauling or unloading the materials at the site.

- INFORMATION GIVEN PRIOR TO AWARD

Oral explanations, instructions and interpretations given to bidders prior to award of contract will not be binding. It is the ACHD’s intent to provide all bidders equal opportunity to access and acquire all available pertinent information necessary to formulate a responsive bid. Any information, specifications, plans, data, or interpretations that ACHD discovers is lacking and may be important to all bidders will be furnished in the form of an addenda, the receipt of which shall be acknowledged by the bidder.

- PROPERTY OWNER COORDINATION

Contractor shall coordinate items of work affecting adjacent property owners, including but not limited to landscape removal and replacement, sprinklers, irrigation facilities, fencing, sod care, and trees.  Property access coordination shall be conducted according to General Condition 49 – PROPERTY ACCESS POINTS.

- PAY QUANTITIES

# By the 25th of each month the Contractor shall supply quantities that are to be paid for that month. Each bid item shall be accompanied with the appropriate backup documentation on how the pay quantity was calculated. This shall include such documentation as, but not limited to; stationing, location, tallied weight tickets, sketch of calculated areas, and/or any other appropriate documentation to verify the pay quantity request. ACHD will verify quantities prior to remitting payment. Items failing to have the appropriate backup supporting documentation will not be paid until the information is supplied. The cost associated with providing the above outlined documentation shall be considered incidental to the project and no additional compensation will be granted.

# All Pay quantities shall also adhere to the criteria outlined in General Condition 41 – MANNER OF PAYMENT.

- POWER SUPPLY

If indicated on the plans or directed by ACHD, the contractor shall install a three-wire electrical service to be used at 120/240 volts, single phase, 60 hertz AC between the power supply and the service cabinet.  The contractor shall install a junction box a maximum of two (2) feet from the power supply.  The distance from the power source to the service cabinet shall not exceed 300’ without approval from the ACHD Signal Coordinator.  The contractor shall coordinate with Idaho Power for the power connection location.  The District shall be responsible for all Idaho Power fees related to power supply and connection.  The contractor shall have Idaho Power submit paperwork to the ACHD Utility Coordinator for the connection a minimum of sixty (60) working days prior to activating the new power supply.  The contractor shall be responsible for all electrical permit fees.

- SHOP DRAWING REVIEW

Plans show details of all structures, lines, grades, typical cross sections of the roadway, location and design of all structures and a summary of items appearing on the proposal. The Contractor shall keep one set of plans available on the work site at all times.

The Contractor will supplement the plans by such working drawings necessary to adequately control the work. Working drawings shall be submitted to the Engineer for review in advance of the start of the proposed work.  The Contractor shall allow sufficient Engineer review time and corrections resubmitted by the Contractor without impacting the approved construction schedule.  The drawings shall be on sheets measuring 22 in. x 34 in. Unless otherwise stated in the contract, the Engineer will require up to 15 business days from the date the submittals or re-submittals are received until they are returned to the Contractor.  No extension to contract time will be given due to re-submittals or the failure of the Contractor to provide any information necessary in a timely fashion.  If necessary, drawings will be signed and sealed by a Professional Engineer licensed to practice in the State of Idaho.

This review time will increase if the drawings submitted do not meet the contract requirements or contain sufficient details.

For structures, six sets of working drawings shall be furnished and shall include stress sheets, shop drawings, erection plans, coffer dam plans, bending diagrams for reinforcing steel, or any other supplementary plans or similar data required of the Contractor.  All structural drawings provided shall be sealed by a Professional Engineer licensed to practice in the State of Idaho.

Working drawings shall be approved before beginning the work covered by the submitted drawings. Such approval is only for general conformance with the information given in the contract documents and does not relieve the Contractor of responsibility for correctness of the details and dimensions, nor does it waive any requirements of the specifications.

The contract price shall include the cost of furnishing all submitted shop and/or working drawings.

- STORMWATER FLOWS

The Contractor is responsible for transmitting existing stormwater and irrigation flows during construction.  All costs associated with transmitting existing flows, including flows from the temporary water quality best management practices on the project, shall be considered incidental to other items of work and no additional compensation will be made.

- STORM WATER MANAGEMENT

Contractor shall implement Best Management Practices (BMPs) controls to protect storm sewer systems and Waters of the United States located on or near project limits.  Contractor shall follow the requirements of the Idaho Pollutant Discharge Elimination System (IPDES) Construction General Permit (CGP) and/or ACHD’s Construction Site Discharge Control (CSDC) Program as required. The contractor is required to implement and maintain BMPs through life of contract.  The removal of all temporary construction BMPs after completion of all required work to final stabilization is considered incidental to this project.

The ACHD CSDC Plan shall meet the CSDC Program requirements listed in Sections 8305 to 8306 of the ACHD Policy Manual

https://www.achdidaho.org/Documents/ACHDpolicyManual/7000to9000/Section8300_ConstructionSiteDischargeControlProgram.pdf.

The CSDC Plan shall include measures to protect the existing ACHD storm drain systems inside and outside the project boundaries from mud, dust, debris, and other pollutants generated or transported due to this project. All measures will also be implemented to protect any “Waters of the United States” inside or outside of the project boundaries.  “Pollutants” are defined in Section 8302 of the ACHD Policy Manual.

The IPDES CGP Stormwater Pollution Prevention Plan (SWPPP) shall meet the CGP requirements. The CGP can be found on the Idaho Department of Environmental Quality (IDEQ) website at: https://www2.deq.idaho.gov/admin/LEIA/api/document/download/16509.

BMPs for controlling pollutant transport from the construction site and activities can be found in a number of publications and locations including, but not limited to:

- Idaho Department of Environmental Quality, Idaho Catalog of Storm Water Best Management Practices at: https://www.deq.idaho.gov/water-quality/wastewater/storm-water/

or Phone: (208) 373-0502

- United States Environmental Protection Agency – Region 10, Resources and Tools at: https://www.epa.gov/npdes/construction-general-permit-resources-tools-and-templates

or Phone: (800) 424-4372

- Idaho Transportation Department, Best Management Practices (BMP) Manual at: https://itd.idaho.gov/env/?target=resources or Phone: (208) 334-8476

Payment for this item will be made under the appropriate bid item numbers.

- SUBGRADE

# It shall be the Contractor’s responsibility to maintain the integrity for the exposed subgrade. Degradation due to his operations or weather shall be remedied under the guidance of a geotechnical engineer.

# All costs to maintain and repair including, geotechnical engineering, etc. shall be incidental to the project and no additional compensation will be made.

- TRAFFIC CONTROL

Contractor shall furnish ACHD with a detailed traffic control plan based on their anticipated approach to construction activities. This plan and implementation will be the responsibility of the contractor. Final approval of the traffic control plan and revisions thereof must be granted by ACHD prior to implementation.  It will be the sole discretion of the ACHD Resident Engineer, or designated representative, to determine if the advanced construction signing plan contained in the contract documents shall be approved as a traffic control plan.

Traffic control plans and/or advanced construction signing plans that may be contained within the project plans are primarily for bidding purposes only.

Local and emergency vehicle access shall be maintained at all times, unless otherwise indicated in the contract documents.  All traffic control items not specifically listed in the bid schedule shall be incidental to this contract, and no additional compensation will be made. Short term lane closures shall be only as approved by the Resident Engineer.

- TACK COAT

Tack Coat is required between lifts of asphalt and between existing and overlayed lifts of asphalt. Tack Coat shall be CSS-1 Diluted Emulsified Asphalt with an application rate of 0.12 Gallons/SY. Tack Coat shall be incidental to other pay items and no additional payment made.

- TRENCHING

All excavation and trenching shall meet OSHA requirements.  Trenches shall be backfilled and properly compacted, closed at the end of each day, and barricaded, as needed.

- WARNING AND REGULATORY SIGNS

Existing warning and regulatory signing shall be retained and protected throughout the project limits unless otherwise noted.  This may require multiple relocations, which will be incidental to other bid items.  Existing warning and regulatory signs that are designated for removal and replacement and do not conflict with construction traffic control signs shall be maintained during construction and, if necessary, relocated to locations where the signs are visible to traffic and serve their original purpose.

- CONSTRUCTION OF IRRIGATION IMPROVEMENTS

All irrigation improvements shall be constructed during the irrigation off-season, generally October 15th to March 15th. All irrigation improvements shall be completed prior to March 15th, 2023. Contractor shall meet all requirements stated in License Agreements obtained from Irrigation Districts and/or Irrigation users.

- WORKING HOURS

Unless otherwise specified here the Regular working hours for this contract shall adhere to the criteria outlined in General Condition 40 – WORK HOURS/TIME OF COMPLETION.

If work hours are to be modified indicate the modification here – If no change to GC delete this text and turn off the highlight above.

- QUALITY CONTROL/QUALITY ASSURANCE

SECTION 100 – General Conditions

Add the following:

Ada County Highway District Quality Control and Quality Assurance Procedure

Definitions

Quality Assurance (QA) is defined as the process or set of processes used to measure and assure the quality of a product and/or workmanship.  This encompasses ACHD’s oversight of the Developer’s/Contractor’s processes and materials.  This includes review of the inspector, sampler, tester and laboratory qualifications (per ACHD Independent Assurance Program defined below); verifying the results of the quality control and process control testing; and inspecting for conformance to plans and specifications.

Quality Control (QC) is defined as the actions necessary to produce quality workmanship and materials.  QC includes, but should not be limited to, inspection of the production and placement operations, quality control and process control testing and inspection of the finished product.  QC is the responsibility of the Developer/Contractor.  No separate payment will be made for this work.  If quality control testing is not being performed at the required frequency, ACHD may stop production of work until the tests are supplied, or ACHD determines the work can proceed.

Quality Control Testing and Process Control Testing are defined as the testing and inspections conducted by the Developer/Contractor to determine if the construction materials  have been produced and placed in compliance with the project specifications and applicable standards.  Process control testing is conducted to demonstrate that the construction materials being produced and used continue to meet the requirements for the product.

Independent Assurance Program (IAP) is the process provided by ACHD, through trained and certified staff, to ensure that all testing is performed correctly and testing equipment is functioning and calibrated properly.

Acceptance is determined by the QA test results, in combination with the assurance that the Developer/Contractor has satisfied the specification requirements for materials quality and workmanship, with the exception of Superpave Hot Mix, as specified below.  In addition to the specified testing results, visual inspection of the end product is also taken into consideration in determining acceptance.  Acceptance will be determined by ACHD.  All samples required for QA testing will be supplied by the Developer/Contractor to ACHD.  Prior to obtaining a sample for QA testing, ACHD shall be notified of when and where the sample will be taken from.  ACHD will determine if it needs to witness the obtaining of the QA sample.  Superpave Hot Mix acceptance shall be based on ACHD’s supplement to the ISPWC Section 814 under Part 4.2.

Construction Testing and Inspection Responsibilities

Quality Control

- The Developer/Contractor shall be responsible for the Quality Control of all construction processes and materials quality.  The Developer/Contractor will be responsible for quality throughout the construction process.  Therefore, the Developer/Contractor must ensure that the materials and workmanship provided by themselves, Subcontractors, Suppliers, and Producers meet all pertinent specifications.  All tests listed under Quality Control in the Minimum Testing Frequency Table are required for quality control, however only tests listed as Acceptance shall be used in determining pay.

- The Quality Control laboratory must be approved by the Idaho Transportation Department.  All testing personnel and facilities used by the Developer/Contractor must be currently certified by the Western Alliance for Quality Transportation Construction (WAQTC) and by the ACHD Independent Assurance Program.  All test results must be supplied to ACHD the next working day after the test was performed.  All Quality Control tests will be supplied to ACHD directly from the lab performing the tests.  Tests results shall have the ACHD project/permit number listed and be sequential.

- Quality Control Testing must occur in a random frequency and at a minimum, at the intervals specified in the table provided in the corresponding ISPWC sections.  The Developer/Contractor shall determine the random locations.  Upon request, documentation must be supplied showing how the random numbers/locations were determined by the Developer/Contractor. All random sampling for ACHD will be done by the Developer/Contractor and witnessed by an ACHD representative.

- For gradation testing by the Developer/Contractor during production, each sample size shall be taken per AASHTO T2, except the sample size shall be doubled.  The sample obtained shall be split in accordance with AASHTO T248, and half of it shall be marked with an ID number and tested by the Developer/Contractor.  The other half of the sample shall be sealed in a canvas sack or plastic bucket, and marked with the ID number, stored in a weather protected container, and reserved for use in retesting if needed.  Material shall be retained until written notification is given by ACHD that it can be discarded.

- Acceptance, gradation, binder content and volumetrics for plant mix pavement and/or Superpave Hot Mix asphalt shall be performed on the loose mix sampled from the roadway, haul unit, or an approved sampling method at the Hot Plant. At the start of each project the location for sampling shall be determined by ACHD and all samples for the project shall be obtained from that determined location.  The hot mix sample used for asphalt binder content, gradation and volumetric testing shall be doubled in size.  The sample shall be split in accordance with AASHTO R47 and half of the sample shall be used for the appropriate testing.  The other half of the sample shall be sealed in a box, marked with the sample ID number, stored in a weather protected enclosure, and reserved for use in retesting if needed. Material shall be retained until written notification is given by ACHD that it can be discarded.

Quality Assurance

- ACHD shall oversee the Developer’s/Contractor’s fulfillment of the QC requirements, and independently verify that the QC test results being submitted by the Developer/Contractor are representative of the workmanship and product quality.  ACHD will also be responsible for determining general project acceptance based on conformance to the approved plan and specifications.

- ACHD will perform random Quality Assurance testing at the intervals specified in Minimum Testing Frequency table.

- Acceptance of material will be based on the Quality Assurance test performed by ACHD.  If no QA test is performed, acceptance will be based on the QC tests results and inspections as determined by ACHD.

- ACHD will perform random inspections of material storage and handling practices.

Dispute Resolution

- The dispute resolution process is to resolve differences between the Developer/Contractor and ACHD when a disagreement regarding the test results occurs.

- When a failing QA test occurs, production may be suspended until the differences are resolved and ACHD is satisfied.

- ACHD may run one or more tests of the split material from the QC process to help resolve differences.

- When ACHD and the Developer/Contractor are unable to resolve the differences, a Third Party prequalified independent lab shall be brought in to verify testing.

- ACHD will hire the Third Party independent lab.  If ACHD tests are found to coincide with the independent lab, ACHD will not grant additional contract time due to any delays in production and the Developer/Contractor shall bear the costs associated with the Third Party resolution.  Likewise, if the Developers/Contractors tests are found to coincide with the independent lab, ACHD will bear the costs associated with Third Party resolution and will grant additional time for any delay caused by this additional testing.

- For hot mix density dispute testing, cores shall be obtained by the Third Party from the same locations as the nuclear gauge tests.

Independent Assurance Program

- ACHD practices the standards of the Western Alliance for Quality Transportation Construction (WAQTC).

- ACHD Independent Assurance personnel may observe testers and verify that the equipment and techniques used for the required testing are calibrated and performing accurately, per WAQTC.  During construction, it may be necessary for an IAP representative to verify the reliability of the tester by witnessing sampling and testing, and by splitting samples and comparing results.

- All independent testing laboratories to be used shall be pre-qualified by ACHD prior to construction.  To request an inspection and certification, contact the ACHD Lab Coordinator at 387-6310 to schedule.  Laboratories that are currently certified by the Idaho Transportation Department (ITD) can provide ACHD with their current certification as confirmation of qualification.  In the event that the lab is ITD certified; a facility inspection by ACHD IAP personnel may still be required.

Price Adjustment for Non- Compliant Materials or Products

- When material fails to meet the required specifications, they will be subject to a price adjustment or rejection.  ACHD may also request a lengthened warranty period.

- ACHD will, in its sole, discretion determine the price adjustment or rejection.

- The determined price adjustment or rejection of material shall be applied to the quantity of material that is represented by the non-compliant test results, as outlined in the Minimum Testing Frequency table.

Minimum Testing Requirements

- The requirements outlined herein are the established minimum acceptance requirements for materials used in standard applications and paid for under standard bid items. For special provision items, material used in non-standard, non-roadway, temporary applications, or small quantities of materials, alternative materials acceptance requirements will be determined as discussed herein or as specified in the contract documents, or as otherwise approved by ACHD. Material placed without appropriate testing shall be subject to a price deduction or rejected as determined by ACHD.

- Minimum testing frequencies are included in the Minimum Testing Frequency table. The Developer/Contractor frequencies may only be altered by Change Order approved by ACHD.  ACHD may elect to increase its own testing frequency at any time. Testing frequency should be increased when accepting material from newly developed sources or those with varying results.

- Material source approval requirements are not listed in this document.  All fill and aggregate material imported to the project must be obtained from ACHD or ITD approved materials sources.

- Based on inspection and without regard for testing frequency, ACHD may isolate and reject obviously defective material.

- When the material is not listed in the Minimum Testing Frequency table and not identified in the ISPWC, acceptance shall be determined by ACHD.

- When the material is required by the Contract to meet a given specification, such as ASTM or AASHTO, acceptance of material will, at a minimum, require a manufacturer’s certification.  A partial list of such material is outlined in the List of Miscellaneous Material Accepted on the Basis of Manufacturer’s Fabricator’s Certification.

Small Quantities

- ACHD may accept small quantities of certain materials without sampling and testing.  The determination to accept materials using this provision rest solely with ACHD.

- The following are not eligible for small quantity acceptance:

    - Concrete with a specified strength of greater than 3000 psi.

    - Paving on the roadway with quantities above 100 ton.

- Materials may be accepted as a small quantity if the estimated quantity is less than the minimum QC testing frequency.  The small quantity exception may be used on pavement items such as; small patches, utility repairs, pavement placed outside the traveled way (driveways, approaches, mailbox turnouts, asphalt sidewalk and curb), and temporary pavement.

- The minimum requirements that must be met for small quantities includes; approved sources, mix design, material certifications, inspection, and a core for density acceptance of mainline and intersection paving less than 100 tons.

ROADWAY SPECIAL PROVISIONS – MODIFY BELOW -

- 202.4.1.A.1 – EXCAVATION (PLAN QUANTITY)

ON PAGE 14 OF SECTION 202 OF THE ISPWC, PART 4.1.A, replace the entire section with the following:

This item shall be paid for by the cubic yard on a plan quantity basis with no final measurement, for which the price and payment shall constitute full compensation for excavating, loading, hauling, spreading, blending, shaping, drying, watering and compacting excavated material that is acceptable for the use as on-site borrow for embankment fill, and for all tools, labor, equipment, materials and incidentals necessary to complete the work and all appurtenances not itemized on the Bid Schedule.

Payment for this item will be made under:

202.4.1.A.1 – Excavation (Plan Quantity) 	……………………………………Per Cubic Yard

- 202.4.5.A.1 – UNSUITABLE MATERIAL EXCAVATION

ON PAGE 17 OF SECTION 202 OF THE ISPWC, PART 4.5.A, add the following:

ITEM 29, “VARIATIONS IN QUANTITIES”, ON PAGE GC15 OF THE ACHD GENERAL CONDITIONS, SECOND PARAGRAPH, shall not apply to the 202.4.5.A.1 bid item.

Payment for this item will be made under:

202.4.5.A.1 – Unsuitable Material Excavation …………………………………..………………..	Per Cubic Yard

- 601.4.1.O.1 – IRRIGATION DITCH, SIZE 4’ WIDE X 3’ DEEP

ON PAGE 11 OF SECTION 602 OF THE ISPWC, PART 4.1.O, add the following:

This item shall include all costs associated with constructing an open irrigation ditch of similar size and shape as existing in locations shown on the project plans.  Finished ditch will be neat in appearance graded to match flow line elevations specified on the plans. The Irrigation Ditch shall be Type A, constructed per ISPWC SD-621 Drawing

Payment for this item will be made under:

602.4.1.O.1 – Irrigation Ditch, Size 4’ Wide x 3’ Deep………………………………………….Per Lineal Feet

- 602.4.1.K.1.12B – 12” DIA. GRAVITY IRRIGATION SLIDE & GATE TYPE C-8

ON PAGE 11 OF SECTION 602 OF THE ISPWC, PART 4.1.K, add the following:

This item shall include all costs associated with furnishing and installing canal gate in the irrigation boxes at the locations shown on the construction plans. The slide gates shall be waterman C-8 style or approved equal.

Payment for this item will be made under:

602.4.1.K.1.12B – 12” Dia. Gravity Irrigation Slide & Gate, Type C-8…………....Per Each

- 602.4.1.M.1 – CONCRETE IRRIGATION BOX- 5’x5’ O.D.

ON PAGE 11 OF SECTION 602 OF THE ISPWC, PART 4.1.M, add the following:

This item shall include all costs associated with construction of an irrigation box as specified on plans, with 6-inch thick concrete walls and 8-inch thick concrete bottom placed at the location and grades detailed on the plan sheets. 5’x5’ Dimensions are outside dimensions.

Included in this item, but considered incidental to the construction of the box and for which no separate payment will be made, are the following:

Furnishing and installing Grade 60 No. 4 reinforcing steel to be placed on 1-foot centers both ways within the walls and bottom of the irrigation boxes.

Furnishing and placing four 6"x1/2" galvanized steel anchor bolts with nuts and washers for securing the lid.

Furnishing and placing a 4-pound expanded steel grating lid with 2"x1/8" steel flat bar edging.

All excavation, backfill and compaction to grade are included in this item.

Concrete shall be All Class CL-3000B.

Payment for this item will be made under:

602.4.1.M.1 – Concrete Irrigation Box – 5’x5’ O.D…………………………………………………………….Per Each

- 706.4.1.F.1 – CONCRETE DRIVEWAY APPROACH

ON PAGE 7 OF SECTION 706 OF THE ISPWC, PART 4.1.F, add the following:

This item includes all labor, equipment and materials necessary to construct a 6” thick concrete driveway approach over 4” of ¾” minus crushed aggregate base material as shown on project plans. The contractor shall limit any driveway closures to 3-days without prior approval from ACHD and the Property Owner.

Excavation and ¾” minus crushed aggregate base material shall be paid separately.

Payment for this item will be made under:

706.4.1.F.1 – Concrete Driveway Approach	Per Square Yard

- 706.4.1.E.1 – CONCRETE SIDEWALK

ON PAGE 7 OF SECTION 706 OF THE ISPWC, PART 4.1.E, add the following:

This item includes all labor, equipment, and materials necessary to construct concrete sidewalks in locations identified on the project plans. Sidewalk cross slope shall not exceed 2% maximum and should be constructed per SD-709.

Excavation and ¾” minus crushed aggregate base material shall be paid separately.

Payment for this item will be made under:

706.4.1.E.1 – Concrete Sidewalk	Per Square Yard

- 706.4.1.G.1 – CONCRETE REPAIR

ON PAGE 7 OF SECTION 706 OF THE ISPWC, PART 4.1.G, add the following:

This item includes all labor, equipment, and materials necessary to construct 6” thick concrete driveway repairs over 4” of ¾” minus crushed aggregate base material. All other concrete repair items will consist of 5” thick concrete over 4” of ¾” minus crushed aggregate base material.

Excavation, compaction, and ¾” minus gravel base material shall be considered incidental to this pay item.

Payment for this item will be made under:

706.4.1.G.1 – Concrete Repair	Per Square Yard

- 706.4.1.H.1 – PEDESTRIAN RAMP W/ DETECTABLE WARNING DOMES

ON PAGE 7 OF SECTION 706 OF THE ISPWC, PART 4.1.H, add the following:

This item shall include all costs associated with completion of the required concrete pedestrian ramps. All pedestrian ramps shall have a minimum four (4) foot wide throat, if not specified on the project plans. All pedestrian ramps shall include full width prefabricated truncated domes as shown on the project plan sheets. Refer to ISPWC standard drawings SD-712* and ACHD Supplementals for additional details. All pedestrian ramps shall be constructed in full compliance with current requirements of the Americans with Disabilities Act (ADA). Truncated domes are considered incidental to this bid item and no additional compensation will be granted.

Payment for this item will be made under:

706.4.1.H.1.A.A – Pedestrian Ramp w/ Detectable Warning Domes, Type SD-712A……………….Per Each

706.4.1.H.1.C.C3 – Pedestrian Ramp w/ Detectable Warning Domes, Type SD-712C………………Per Each

706.4.1.H.1.G.G – Pedestrian Ramp w/ Detectable Warning Domes, Type SD-712G………………Per Each

- 801.4.1.B.1.6 – 6” MINUS UNCRUSHED AGGREGATE BASE

ON PAGE 2 OF SECTION 801 OF THE ISPWC, PART 2.1 – DESCRIPTION, add the following:

- The material shall have a minimum R-value of 70.

ON PAGE 6 OF SECTION 801 OF THE ISPWC, PART 4.1.B, add the following:

The pay quantity for this item was calculated using a unit weight of 135 pcf at a moisture content of 7 percent.

Payment for this item will be made under:

801.4.1.B.1.6 – 6” Minus Uncrushed Aggregate Base	Per Ton

- 802.4.1.A.1 – CRUSHED AGGREGATE FOR BASE - TYPE I (PLAN QUANTITY)

ON PAGE 7 OF SECTION 802 OF THE ISPWC, PART 4.1, add the following:

Material will be calculated using plan quantity.

Payment for this item will be made under:

802.4.1.A.1 – Crushed Aggregate Base -  Type I (Plan Quantity)….	Per Cubic Yard

- 814.4.1.A.1 – PLANT MIX PAVEMENT – SUPERPAVE SP-3

Division 800 –

- Section 810 – Plant Mix Pavement, Part 1.2 – Related Sections, Paragraph E will be deleted in its entirety

- Section 810 – Plant Mix Pavement, Part 2.1 – Hot Mix Asphalt Design, Paragraph D will be deleted in its entirety

- Section 814 – Superpave Plant Mix Asphalt shall be added in its entirety

SECTION 814

SUPERPAVE PLANT MIX ASPHALT

PART 1	GENERAL

  - SECTION INCLUDES

- This work consists of constructing one or more courses of Superpave Plant Mix pavement in accordance with these specifications and in reasonably close conformity to the lines, grades, thicknesses, and typical cross section(s) shown in the Contract Documents, or as established.

  - RELATED SECTIONS

- Section 803 – Plant Mix Aggregate

- Section 805 – Asphalt

- Section 806 – Tack

- Section 810 – Plant Mix Pavement

  - REFERENCES

- AASHTO Standard Specifications for Transportation and Methods of Sampling and Testing

- WAQTC – Western Alliance for Quality Transportation Construction

PART 2	MATERIALS

2.1	CLASSIFICATION

- The Superpave HMA shall be composed of a combination of aggregate, mineral filler (if required), and performance graded (PG) asphalt binder material. The Contractor shall furnish a job mix formula (JMF) and a HMA pavement that complies with the following requirements. Any JMF dated more than 1-year from the date of submittal will either require updated specific gravities from the original crush, or a new JMF. Updated specific gravities shall not alter the JMF target values out of tolerance; otherwise a new JMF will be required.

  - HOTPLANT, AGGREGATE, MIX DESIGN REQUIREMENTS and PRODUCTION LIMITS

- Hotplants shall be capable of producing a printed ticket every 15 minutes during production paving that will be submitted daily to the Contract Administrator and include the following information:

  - Date

  - Time Printed

  - Project Identity (i.e., name, number, location)

  - Material Quantity

- Aggregate for all mixes, except SP2, as a minimum shall be provided in two separate stockpiles. Aggregate shall be crushed stone or crushed gravel of such gradation that when combined with other required aggregate fractions and fillers, in proper proportion, the resultant mixture meets the gradation required under the composition of mixture for the specific class under contract.

- The fractions for the mixture shall be sized, graded, and combined in such proportions that the resulting blend conforms to the grading requirements as defined in Table 1 below.

- Aggregates shall meet the requirements of Section 803 – Plant Mix Aggregates with the exception of Table 1 through 4.

| Table 1 SUPERPAVE AGGREGATE DESIGN BANDS and VMA TOLERANCES |  |  |  |
| --- | --- | --- | --- |
| SIEVE SIZE | NOMINAL MAXIMUM SIZE |  |  |
|  | 3/4 in. | 1/2 in. | 3/8 in. |
|  | PERCENT PASSING |  |  |
| 1 in. | 100 |  |  |
| 3/4 in. | * 90-100 | 100 |  |
| 1/2 in. | 90 max | * 90-100 | 100 |
| 3/8 in. | * 52-80 | 90 max | * 90-100 |
| No. 4 | - | - | 90 max |
| No. 8 | * 23-49 | * 28-58 | * 32-67 |
| No. 200 | * 2.0-8.0 | *2.0-10.0 | * 2.0-10.0 |
| VMA, % Minimum | 13.0 | 14.0 | 15.0 |
| PRIMARY CONTROL SIEVE (PCS) CONTROL POINT FOR MIXTURE NOMINAL MAXIMUM AGG SIZE ** |  |  |  |
| Primary Control Sieve | No. 4 | No. 8 | No. 8 |
| PCS Control Point (% passing) | 47 | 39 | 47 |

- * Denotes the sieves that will be used for mix design control points and quality analysis sieves for Class SP2 mixes.

- ** The combined aggregate gradation shall be classified as coarse graded when it passes below the PCS control point as defined in table 1.  All other gradations shall be classified as fine graded. (This classification is based on the Contractors JMF and not individual gradation tests.)

| TABLE 2 SUPERPAVE MIXTURE REQUIREMENTS |  |  |  |
| --- | --- | --- | --- |
| Minimum Use | Temporary Paving | Local, Arterials & Collectors |  |
| Quality Characteristics | SP2 | SP3 | SP5 |
| Design ESALs (million) (1) | < 1 | 1 - <10 | 10 - < 30 |
| Gyratory Compaction Gyrations for Ndes | 50 | 75 | 100 |
| Relative density, %Gmm@Ndes | 96.0 | 96.0 | 96.0 |
| Air Voids, %VA | 4.0 | 4.0 | 4.0 |
| Dust to Binder Ratio Range, DP (2) | 0.6-1.4 | 0.6-1.4 | 0.6-1.4 |
| Voids Filled with Asphalt, % VFA range | 65 – 78 | 65 – 75 | 65 – 75 |
| Idaho Degradation, max loss, % | 5.0 |  |  |
| Ethylene Glycol, min retained, % | 90 |  |  |
| R-Value | 80 or more |  |  |
| Sodium Sulfate Soundness, max loss after 5 cycles, % (3) | 12 |  |  |
| LA Wear – AASHTO T-96, Max % loss | 35 | 30 | 30 |
| Fractured Face - 1 fracture/2 fracture, % Min (4) | 65/- | 75/60 | 95/90 |
| Uncompacted void content of fine agg, % Min | 40 | 40 | 45 |
| Sand Equivalent, % Min | 35 | 40 | 45 |
| Flat & Elongated, % Max | 10 | 10 | 10 |

- The anticipated project traffic level expected on the design lane over a 20-year period. Regardless of the actual design life of the roadway, determine the design ESALs for 20 years.

- For No. 4 nominal maximum size mixtures, the dust-binder-ratio is 0.9 to 2.0. If the aggregate gradation passes beneath the PCS Control Point specified in Table 1, the allowable dust-to-binder ratio range may increase from 0.6-1.4 to 0.8-1.6.

- Perform sodium sulfate soundness testing when requested by the Engineer.

- 95/90 denotes that 95% of the coarse aggregate has one fractured face and 90% has two or more fractured faces.

| Table 3 SUPERPAVE PRODUCTION REQUIREMENTS |  |
| --- | --- |
| Quality Characteristics | SP2, SP3,SP5 |
| Asphalt Binder, % PBe | JMF value ± 0.4 |
| Laboratory Air Voids, % Va | 4.0 ± 1.0 |
| Voids in Mineral Agg, VMA | Per Table 4 |

| Table 4 SUPERPAVE AGGREGATE GRADATION & VMA TOLERANCES - PRODUCTION |  |  |  |
| --- | --- | --- | --- |
| SIEVE SIZE | TOLERANCES FROM JMF |  |  |
|  | 3/4 in. | 1/2 in. | 3/8 in. |
| 1 in. – No.4 | JMF value ± 6.0% |  |  |
| No. 8 – No. 30 | JMF value ± 5.0% |  |  |
| No. 50 – No. 100 | JMF value ± 4.0% |  |  |
| No. 200 | JMF value± 2.0% |  |  |
| VMA, % min | 13.0 | 14.0 | 15.0 |

- Please see ACHD’s QC/QA table for sampling requirements

- Tolerances cannot be outside of design band

2.3	ASPHALT

- Asphalt Binder shall meet the requirements of Section 805 – Asphalt.

  - Asphalt to be of the type and grade called for in the Contract Documents.

  - Asphalt will be accepted at the point of delivery.

  - Unless otherwise permitted, all asphalt for a specified project shall be furnished by one (1) supplier.  If a change of supplier for asphalt is proposed, or if blending of plant mix asphalt from more than one supplier is proposed, mix design testing and verification are required as conditions of approval.

2.4	ANTI-STRIPPING ADDITIVE

- Anti-stripping additive shall be added by the binder supplier.

- All Superpave Plant Mixes shall use a minimum 0.5% approved liquid anti-stripping additive by weight of asphalt.

2.5	TEST METHODS

- Sieve Analysis of Fine and Coarse Aggregates – AASHTO T 27.

- Materials Finer than No. 200 sieve in Mineral Aggregates by Washing –AASHTO T 11.

- Preparing and Determining the Density of HMA Specimens by Means of the          Superpave Gyratory Compactor – AASHTO T 312.

- Percentage of Fracture in Coarse Aggregate – AASHTO TP 61.

- Plastic Fines in Graded Aggregate and Soils by Use of the Sand Equivalent Test –AASHTO T 176.

- Flat and Elongated Particles in Coarse Aggregate – ASTM D 4791

- Theoretical Maximum Specific Gravity and Density of HMA Paving Mixtures – AASHTO T 209.

- Bulk Specific Gravity of Compacted HMA using Saturated Surface-Dry Specimens – AASHTO T 166.

- Sampling Bituminous Paving Mixtures – AASHTO R 97.

- Sampling Bituminous Materials – AASHTO T 40.

- In Place Density of HMA – AASHTO T 355.

- Determining the Asphalt Binder Content of HMA by Ignition method – AASHTO T 308.

- Bulk Specific Gravity of compacted HMA mixtures using saturated surface-dry specimens – AASHTO T 166.

- Mechanical analysis of extracted aggregate – AASHTO T 30

- Standard Method of Test for Quantitative Extraction of Asphalt Binder from HMA - AASHTO T 164

- Standard Specification for Performance-Graded Asphalt Binder – AASHTO M 320

- Standard Specification for Superpave Volumetric Mix Design – AASHTO M 323

- Standard Practice for Recovery of Asphalt Binder from Solution by Abson Method – AASHTO R 59

PART 3	WORKMANSHIP

3.1	Workmanship shall meet the requirements of Section 810, Part 3. Including, but not limited to, mixing plant, sampling devices, all equipment, paver, rollers, mix design approval and weather limitations and cutoff dates. Pavement shall be compacted to a range between 92% - 96% of the theoretical maximum value from the JMF on the mat. Recycled plant mix (RAP) will be allowed up to 30% by weight of binder as outlined in the requirements of Section 810, Part 2.5

PART 4	MEASUREMENTS AND PAYMENT

  - Measurement and payment shall meet the requirements of Section 810, Part 4 and unless otherwise specified in the contract documents, acceptance of the Superpave plant mix and the incentive/disincentive payment will be in accordance with section 4.2.

A.    Incentive/Disincentive payments will not be calculated for quantities under 1500 tons. Failing tests are subject to rejection or pay reduction as determined by Engineer.

  - Acceptance, Pay Factors & Incentive/Disincentive Payment.  For projects not funded by ACHD, a pay factor of 1.0 will be used, and material failing to meet the project specifications will be subject to rejection, an extended warranty, or a fee.

- Mix Characteristic Acceptance and Pay Factors

Determine the arithmetic mean,

Where

= Summation

xi  	= Individual test value

n  	= Total number test values

Compute the sample standard deviation, (S)

Compute the upper quality index (Qu).

Where USL = Upper specification limit.

S =	Standard deviation

Compute the lower quality index (QL).

Where    LSL = Lower specification limit.

S   = Standard deviation

Determine PU (percent within the upper specification limit, which corresponds to a given QU) from Table7. If a USL is not specified, PU will be 100.

Determine PL (percent within lower specification limit, which corresponds to a given QL) from Table 7. If a LSL is not specified or the specification is zero (0), PL will be 100.

Determine the Quality Level(QL) (the total percent within the specification limits).

Quality Level(QL) = (PU + PL) – 100

For air voids, each lot will be assigned a pay factor using the following equation:

Table 5

PU or PL Percent within Limits for Positive Values of QU or QL for a given Sample Size (n)

| PWL | n = 3 | n = 4 | n = 5 | n = 6 | n = 7 | n = 8 | n = 9 | n = 10 to 11 | n = 12 to 14 | n = 15 to 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 1.16 | 1.50 | 1.79 | 2.03 | 2.23 | 2.39 | 2.53 | 2.65 | 2.83 | 3.03 |
| 99 | – | 1.47 | 1.67 | 1.80 | 1.89 | 1.95 | 2.00 | 2.04 | 2.09 | 2.14 |
| 98 | 1.15 | 1.44 | 1.60 | 1.70 | 1.76 | 1.81 | 1.84 | 1.86 | 1.91 | 1.93 |
| 97 | – | 1.41 | 1.54 | 1.62 | 1.67 | 1.70 | 1.72 | 1.74 | 1.77 | 1.79 |
| 96 | 1.14 | 1.38 | 1.49 | 1.55 | 1.59 | 1.61 | 1.63 | 1.65 | 1.67 | 1.68 |
| 95 | – | 1.35 | 1.44 | 1.49 | 1.52 | 1.54 | 1.55 | 1.56 | 1.58 | 1.59 |
| 94 | 1.13 | 1.32 | 1.39 | 1.43 | 1.46 | 1.47 | 1.48 | 1.49 | 1.50 | 1.51 |
| 93 | – | 1.29 | 1.35 | 1.38 | 1.40 | 1.41 | 1.42 | 1.43 | 1.44 | 1.44 |
| 92 | 1.12 | 1.26 | 1.31 | 1.33 | 1.35 | 1.36 | 1.36 | 1.37 | 1.37 | 1.38 |
| 91 | 1.11 | 1.23 | 1.27 | 1.29 | 1.30 | 1.30 | 1.31 | 1.31 | 1.32 | 1.32 |
| 90 | 1.10 | 1.20 | 1.23 | 1.24 | 1.25 | 1.25 | 1.26 | 1.26 | 1.26 | 1.27 |
| 89 | 1.09 | 1.17 | 1.19 | 1.20 | 1.20 | 1.21 | 1.21 | 1.21 | 1.21 | 1.22 |
| 88 | 1.07 | 1.14 | 1.15 | 1.16 | 1.16 | 1.16 | 1.16 | 1.17 | 1.17 | 1.17 |
| 87 | 1.06 | 1.11 | 1.12 | 1.12 | 1.12 | 1.12 | 1.12 | 1.12 | 1.12 | 1.12 |
| 86 | 1.04 | 1.08 | 1.08 | 1.08 | 1.08 | 1.08 | 1.08 | 1.08 | 1.08 | 1.08 |
| 85 | 1.03 | 1.05 | 1.05 | 1.04 | 1.04 | 1.04 | 1.04 | 1.04 | 1.04 | 1.04 |
| 84 | 1.01 | 1.02 | 1.01 | 1.01 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| 83 | 1.00 | 0.99 | 0.98 | 0.97 | 0.97 | 0.96 | 0.96 | 0.96 | 0.96 | 0.96 |
| 82 | 0.97 | 0.96 | 0.95 | 0.94 | 0.93 | 0.93 | 0.93 | 0.92 | 0.92 | 0.92 |
| 81 | 0.96 | 0.93 | 0.91 | 0.90 | 0. 90 | 0.89 | 0.89 | 0.89 | 0.89 | 0.88 |
| 80 | 0.93 | 0.90 | 0.88 | 0.87 | 0.86 | 0.86 | 0.86 | 0.85 | 0.85 | 0.85 |
| 79 | 0.91 | 0.87 | 0.85 | 0.84 | 0.83 | 0.82 | 0.82 | 0.82 | 0.82 | 0.81 |
| 78 | 0.89 | 0.84 | 0.82 | 0.80 | 0.80 | 0.79 | 0.79 | 0.79 | 0.78 | 0.78 |
| 77 | 0.87 | 0.81 | 0.78 | 0.77 | 0.76 | 0.76 | 0.76 | 0.75 | 0.75 | 0.75 |
| 76 | 0.84 | 0.78 | 0.75 | 0.74 | 0.73 | 0.73 | 0.72 | 0.72 | 0.72 | 0.71 |
| 75 | 0.82 | 0.75 | 0.72 | 0.71 | 0.70 | 0.70 | 0.69 | 0.69 | 0.69 | 0.68 |
| 74 | 0.79 | 0.72 | 0.69 | 0.68 | 0.67 | 0.66 | 0.66 | 0.66 | 0.66 | 0.65 |
| 73 | 0.76 | 0.69 | 0.66 | 0.65 | 0.64 | 0.63 | 0.63 | 0.63 | 0.62 | 0.62 |
| PWL | n = 3 | n = 4 | n = 5 | n = 6 | n = 7 | n = 8 | n = 9 | n = 10 to 11 | n = 12 to 14 | n = 15 to 18 |
| 72 | 0.74 | 0.66 | 0.63 | 0.62 | 0.61 | 0.60 | 0.60 | 0.60 | 0.59 | 0.59 |
| 71 | 0.71 | 0.63 | 0.60 | 0.59 | 0.58 | 0.57 | 0.57 | 0.57 | 0.57 | 0.56 |
| 70 | 0.68 | 0.60 | 0.57 | 0.56 | 0.55 | 0.55 | 0.54 | 0.54 | 0.54 | 0.53 |
| 69 | 0.65 | 0.57 | 0.54 | 0.53 | 0.52 | 0.52 | 0.51 | 0.51 | 0.51 | 0.50 |
| 68 | 0.62 | 0.54 | 0.51 | 0.50 | 0.49 | 0.49 | 0.48 | 0.48 | 0.48 | 0.48 |
| 67 | 0.59 | 0.51 | 0.47 | 0.47 | 0.46 | 0.46 | 0.46 | 0.45 | 0.45 | 0.45 |
| 66 | 0.56 | 0.48 | 0.45 | 0.44 | 0.44 | 0.43 | 0.43 | 0.43 | 0.42 | 0.42 |
| 65 | 0.52 | 0.45 | 0.43 | 0.41 | 0.41 | 0.40 | 0.40 | 0.40 | 0.40 | 0.39 |
| 64 | 0.49 | 0.42 | 0.40 | 0.39 | 0.38 | 0.38 | 0.37 | 0.37 | 0.37 | 0.36 |
| 63 | 0.46 | 0.39 | 0.37 | 0.36 | 0.35 | 0.35 | 0.35 | 0.34 | 0.34 | 0.34 |
| 62 | 0.43 | 0.36 | 0.34 | 0.33 | 0.32 | 0.32 | 0.32 | 0.32 | 0.31 | 0.31 |
| 61 | 0.39 | 0.33 | 0.31 | 0.30 | 0.30 | 0.29 | 0.29 | 0.29 | 0.29 | 0.29 |
| 60 | 0.36 | 0.30 | 0.28 | 0.27 | 0.27 | 0.27 | 0.26 | 0.26 | 0.26 | 0.26 |
| 59 | 0.32 | 0.27 | 0.25 | 0.25 | 0.24 | 0.24 | 0.24 | 0.24 | 0.23 | 0.23 |
| 58 | 0.29 | 0.24 | 0.23 | 0.22 | 0.21 | 0.21 | 0.21 | 0.21 | 0.21 | 0.21 |
| 57 | 0.25 | 0.21 | 0.20 | 0.19 | 0.19 | 0.19 | 0.18 | 0.18 | 0.18 | 0.18 |
| 56 | 0.22 | 0.18 | 0.17 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.16 | 0.15 |
| 55 | 0.18 | 0.15 | 0.14 | 0.14 | 0.13 | 0.13 | 0.13 | 0.13 | 0.13 | 0.13 |
| 54 | 0.14 | 0.12 | 0.11 | 0.11 | 0.11 | 0.11 | 0.10 | 0.10 | 0.10 | 0.10 |
| 53 | 0.11 | 0.09 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 | 0.08 |
| 52 | 0.07 | 0.06 | 0.06 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 | 0.05 |
| 51 | 0.04 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| 50 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

NOTE:	For negative values of QU or QL, PU or PL is equal to 100 minus the table value for PU or PL. If the value of QU or QL does not correspond exactly to a figure in the table, use the next higher figure.

| Pay Factor | n = 3 | n = 4 | n = 5 | n = 6 | n = 7 | n = 8 | n = 9 | n = 10 to n=11 | n = 12 to n=14 | n = 15 to n=18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.05 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 1.04 | 90 | 91 | 92 | 93 | 93 | 93 | 94 | 94 | 95 | 95 |
| 1.03 | 80 | 85 | 87 | 88 | 89 | 90 | 91 | 91 | 92 | 93 |
| 1.02 | 75 | 80 | 83 | 85 | 86 | 87 | 88 | 88 | 89 | 90 |
| 1.01 | 71 | 77 | 80 | 82 | 84 | 85 | 85 | 86 | 87 | 88 |
| 1.00 | 68 | 74 | 78 | 80 | 81 | 82 | 83 | 84 | 85 | 86 |
| 0.99 | 66 | 72 | 75 | 77 | 79 | 80 | 81 | 82 | 83 | 85 |
| 0.98 | 64 | 70 | 73 | 75 | 77 | 78 | 79 | 80 | 81 | 83 |
| 0.97 | 62 | 68 | 71 | 74 | 75 | 77 | 78 | 78 | 80 | 81 |
| 0.96 | 60 | 66 | 69 | 72 | 73 | 75 | 76 | 77 | 78 | 80 |
| 0.95 | 59 | 64 | 68 | 70 | 72 | 73 | 74 | 75 | 77 | 78 |
| 0.94 | 57 | 63 | 66 | 68 | 70 | 72 | 73 | 74 | 75 | 77 |
| 0.93 | 56 | 61 | 65 | 67 | 69 | 70 | 71 | 72 | 74 | 75 |
| 0.92 | 55 | 60 | 63 | 65 | 67 | 69 | 70 | 71 | 72 | 74 |
| 0.91 | 53 | 58 | 62 | 64 | 66 | 67 | 68 | 69 | 71 | 73 |
| 0.90 | 52 | 57 | 60 | 63 | 64 | 66 | 67 | 68 | 70 | 71 |
| 0.89 | 51 | 55 | 59 | 61 | 63 | 64 | 66 | 67 | 68 | 70 |
| 0.88 | 50 | 54 | 57 | 60 | 62 | 63 | 64 | 65 | 67 | 69 |
| 0.87 | 48 | 53 | 56 | 58 | 60 | 62 | 63 | 64 | 66 | 67 |
| 0.86 | 47 | 51 | 55 | 57 | 59 | 60 | 62 | 63 | 64 | 66 |
| 0.85 | 46 | 50 | 53 | 56 | 58 | 59 | 60 | 61 | 63 | 65 |
| 0.84 | 45 | 49 | 52 | 55 | 56 | 58 | 59 | 60 | 62 | 64 |
| 0.83 | 44 | 48 | 51 | 53 | 55 | 57 | 58 | 59 | 61 | 63 |
| 0.82 | 42 | 46 | 50 | 52 | 54 | 55 | 57 | 58 | 60 | 61 |
| 0.81 | 41 | 45 | 48 | 51 | 53 | 54 | 56 | 57 | 58 | 60 |
| 0.80 | 40 | 44 | 47 | 50 | 52 | 53 | 54 | 55 | 57 | 59 |
| 0.79 | 38 | 43 | 46 | 48 | 50 | 52 | 53 | 54 | 56 | 58 |
| 0.78 | 37 | 41 | 45 | 47 | 49 | 51 | 52 | 53 | 55 | 57 |
| 0.77 | 36 | 40 | 43 | 46 | 48 | 50 | 51 | 52 | 54 | 56 |
| 0.76 | 34 | 39 | 42 | 45 | 47 | 48 | 50 | 51 | 53 | 55 |
| 0.75 | 33 | 38 | 41 | 44 | 46 | 47 | 49 | 50 | 51 | 53 |

- Pay Factors for Gradation (SP-2 only), VMA (SP-3 and SP-5) and Density (all mix classes)

Table 6

Pay Factor for a given Sample Size (n) and Quality Level

- Calculation of Incentive/Disincentive Payment for SP-2 mixes

      - Pay factors for test strips, leveling courses, approaches and miscellaneous paving not placed with mainline paving shall be 1.00.  The Maximum Pay Factor will be 1.05.  If any individual Composite Pay Factor Value falls below 0.85 the maximum Pay Factor Value, the lowest CPF Value.   Material with a Pay Factor less than 0.75 shall be rejected and removed at no cost to the District.

      - A Composite Pay Factor for Plant Mix Aggregate (CPF(PMA)) will be computed as:

a.   (PFAV) (0.3) = CPF(PMA)

b.   PFAV = Weighted average based on quantity of material in each lot.

      - A Composite Pay Factor for Asphalt Binder Content (CPF(ABC)) will be computed as:

a.   (PFAV) (0.3) = CPF(ABC)

b.   PFAV = Weighted average based on quantity of material in each lot.

      - A Composite Pay Factor for Density (CPF(Dens.)) will be computed as follows:

a.   (PFAV) (0.4) = CPF(Dens.)

b.   PFAV = Weighted average based on quantity of material in each lot.

      - Calculation of Incentive/Disincentive Payment.  The incentive/disincentive payment for all Superpave plant mix pavement accepted by the Owner, excluding plant mix pavement for approaches and miscellaneous paving not placed with mainline paving, will be computed using the formula:

a.   B = (A) ((CPF(PMA) + CPF(ABC) + CPF(Dens.)) -1) (Q)

b.   B = Total Incentive/disincentive payment for all Plant Mix Pavement accepted

c.   A = Unit Bid Price

d.   Q = Total Quantity of Plant Mix Pavement accepted

- Calculation of Incentive/Disincentive Payment for SP-3 and SP-5 mixes

      - Pay factors for leveling courses, approaches and miscellaneous paving not placed with mainline paving shall be 1.00.  The Maximum Pay Factor will be 1.05.  If any individual Composite Pay Factor Value falls below 0.85 the maximum Pay Factor Value, the lowest CPF Value.   Material with a Pay Factor less than 0.75 shall be rejected and removed at no cost to the District.

      - A Composite Pay Factor for Air Void (CPF(AIR VOID)) will be computed as:

a.   (PFAV) (0.3) = CPF(AIR VOID)

b.   PFAV = Weighted average based on quantity of material in each lot.

      - A Composite Pay Factor for VMA (CPF(VMA)) will be computed as:

a.   (PFAV) (0.3) = CPF(VMA)

b.   PFAV = Weighted average based on quantity of material in each lot.

      - A Composite Pay Factor for Density (CPF(Dens.)) will be computed as follows:

a.   (PFAV) (0.4) = CPF(Dens.)

b.   PFAV = Weighted average based on quantity of material in each lot.

      - Calculation of incentive/disincentive payment. The incentive/disincentive payment for all Superpave Hot Mix Asphalt accepted by the Owner, excluding plant mix pavement for approaches and miscellaneous paving not placed with mainline paving, will be computed using the formula:

a.   B = (A) ((CPF(AIR VOID) + CPF(VMA) + CPF(Dens.)) -1) (Q)

b.   B =Total incentive/disincentive payment for all Plant Mix Pavement accepted

c.   A = Unit Bid Price

    - Q = Total Quantity of Plant Mix Pavement accepted

PART 5	DISPUTE RESOLUTION SIGNIFICANT DIFFERENCE

  - Table 7 quantifies the significant difference for differing quality assurance measures.

- For Superpave Plant Mix dispute density testing, cores obtained from the same location as the nuclear or non-nuclear gauge test shall be used.

Table 7

| Characteristic | Significant Difference |
| --- | --- |
| Air Voids | 0.5 percent |
| VMA | 0.5 percent |
| Asphalt Content | 0.2 percent |
| Percent Compaction | 1 percent |
| #4 or Larger Sieves | 4 percent |
| #8 to #30 Sieves | 3 percent |
| #50 to #100 Sieves | 2 percent |
| #200 Sieve | 1.0 percent |
| Sand Equivalent | 4 |

  - QUALITY ASSURANCE

      - Quality Assurance/verification of the Contractors testing will be performed by the County.  Quality Assurance test results will not be substituted for acceptance results.

- Quality Assurance results will be used to evaluate the Contractor's Quality Control/acceptance test results.  The data will be evaluated on a cumulative basis and not on a lot by lot basis as follows:

- If the evaluation indicates the test results are consistent (t-test passes), then

the Engineer will combine the Contractor's tests into lots for Quality Analysis. The lots will be used by the Engineer to represent the material produced in Quality Analysis. Tests can only be excluded with approval of the Engineer.  Lot size will be determined by the Engineer.  The following criteria will be used:

    - A lot is based on work shift’s production.

    - Minimum Lot size is 3 tests.

    - If the work shift is represented by less than three tests, the test (s) will                                                   	be combined with the following work shift.

    - If the final work shift is represented by less than three tests, the test (s) 	will be combined with the previous work shift.

- If the evaluation indicates the test results are inconsistent (t-test fails), production shall be stopped. The Engineer will review contractor test procedures, calculations, and documentation to determine the source of the differences. Production will not be allowed to resume until the source of the differences is determined and corrected. If the source of the differences is determined to be caused by the Contractor, the State will not grant additional contract time.

PART 6	HOT MIX DEPTH

6.1	Depth will be based on the average from the cores obtained for the density gauge 		correlations, as outlined in the Minimum Testing Frequency Table for QC/QA.

A.   For newly constructed roadways, roadways that have had the existing plant mix milled the full width, existing plant mix has been removed, or one or more leveling courses are required. If more than one lift of plant mix is placed, the depth will be based on both lifts combined. The following table 8 shall apply.

Table 8

| Actual Pavement Depth Vs. Planned | Payment Adjustment |
| --- | --- |
| Over .55” | No Payment for overage, and remedy action required if under .55” |
| .45” to.55” | 65% Deduct |
| .35” to.45” | 45% Deduct |
| .25” to .35” | 25% Deduct |
| .00” to .25” | 0% Deduct |

B.   Example: 3” of hot plant mix is required per plan.  Cores averaged 3.6”.  700 tons of plant mix was placed. Bid per ton was $60.

Deduct: 	3.00” to 3.25” 	= 	Zero

3.25” to 3.35”	=	700ton x (1-(3.25/3.35)) x $60 x .25% = $313.43 deduct

3.35” to 3.45”	=	700ton x (1-(3.35/3.45)) x $60 x .45% = $547.83 deduct

3.45” to 3.55”	=	700ton x (1-(3.45/3.55)) x $60 x .65% = $769.01 deduct

3.55” to 3.60”	=	700ton x (1-(3.55/3.60)) x $60 x 1.0 % = $588.33 deduct

Total Deduct	= 	$313.43 + $547.83 + $769.01 + $588.33 = $2,218.60 deduct

SP-3 plant mix asphalt pavement shall utilize PG 64-28 liquid asphalt with 0.5% anti-strip. The aggregate shall have a nominal maximum aggregate side of 1/2” (inch).

Payment for this item will be made under:

814.4.1.A.1 – Plant Mix Pavement – Superpave SP-3	Per Ton

- 1003.4.1.B.1 – BIOFILTER BAG (FIBER WATTLE)

ON PAGE 3 OF SECTION 1003 OF THE ISPWC, PART 4.1.B, add the following:

# ITEM 29, “VARIATIONS IN QUANTITIES”, ON PAGE GC15 OF THE ACHD GENERAL CONDITIONS, SECOND PARAGRAPH, shall not apply to the 1003.4.1.B.1 bid item.

1003.4.1.B.1 – Biofilter Bag (Fiber Wattle)	Per Each

- 1006.4.1.C.1 – INLET PROTECTION

ON PAGE 3 OF SECTION 1006 OF THE ISPWC, PART 2.1, add the following:

B. Inlet protection shall be Stormguard Sediment Catch Basin Inserts installed according to manufacturer’s recommendations, or approved equal.

# Payment will be made under:

1006.4.1.C.1 – Inlet Protection	Per Each

- 1103.4.1.B.1 – TRAFFIC CONTROL SIGNS

ON PAGE 7 OF SECTION 1103 OF THE ISPWC, PART 4.1.B, add the following:

Provide and install Class “A” construction signing to direct vehicular traffic in and around the project site, including informational signage, per the MANUAL ON UNIFORM TRAFFIC CONTROL DEVICES, most current edition.  Signs that will be in place longer than 3 days shall be permanent sign installations. Contractor shall bag or cover signs when not in use.  Contractor shall refer to the ATSSA pamphlet called Quality Standards for Work Zone Traffic Control Devices.

# Payment will be made by the square foot of sign acceptably installed, initial installation only.

1103.4.1.B.1 – Traffic Control Signs	Per Square Foot

- SP 06004 – SEEPAGE BED

Description: This item shall include all labor, equipment, and materials to construct and install infiltration beds and all appurtenances at the locations and as detailed on the plans. Infiltration beds shall be constructed in conformance with ACHD Section 8200 Standard Drawing BMP 20.

Materials: Contractor shall construct and install perforated piping, drain rock, filter sand, geotextile fabrics, backfill, observation wells, and appurtenances as detailed on the plans and SD BMP 20.

Measurement and Payment: Payment shall be on a lineal foot basis to the dimensions shown on the plans. Any over excavation and associated drain rock, filter fabric, and backfill material shall be considered incidental to this item and no additional payment will be made.

The cost of any utility service sleeve pipe or solid pipe for services is considered incidental to this bid item and no additional payment will be made.

Shoring boxes may also be required for the construction of this item, and additional compensation will not be made.

Payment for this item will be made under:

SP 06004A – Seepage Bed 6’ D x 12’ Width, 18” Pipe……………….. Per Lineal Foot

SP 06004B – Seepage Bed 5’ D x 12’ Width, 18” Pipe……………….. Per Lineal Foot

SP 06004C – Seepage Bed 4’ D x 15’ Width, 18” Pipe……………….. Per Lineal Foot

SP 06004D – Seepage Bed 10’ D x 12’ Width, 18” Pipe…………….. Per Lineal Foot

- SSP 06013 – STORMWATER MANAGEMENT PLAN PREPARATION AND IMPLEMENTATION

Description: This item shall consist of all work associated with contractor plan preparation and approvals to meet the requirements of the Idaho Pollutant Discharge Elimination System (IPDES) Construction General Permit (CGP) and/or ACHD’s Construction Site Discharge Control (CSDC) Program as required. The contractor is considered an operator having day-to-day control as defined in the CGP; therefore, the contractor is a co-permittee with ACHD in the implementation of the CGP requirements. A Stormwater Pollution Prevention Plan (SWPPP) will be accepted by ACHD in lieu of the CSDC Plan provided that the SWPPP meets the CSDC Program requirements listed in 8305 and 8306 of the ACHD Policy Manual.

https://www.achdidaho.org/Documents/ACHDpolicyManual/7000to9000/Section8300_ConstructionSiteDischargeControlProgram.pdf .

Workmanship: The contractor is responsible for the completion, submittal, and implementation of the ACHD provided SWPPP drawing and narrative, filing of the Notice of Intent (NOI), and filing of the Notice of Termination (NOT).  Access for completing an electronic NOI and NOT can be found on the Idaho Department of Environmental Quality (IDEQ) IPDES E-Permitting website: https://www2.deq.idaho.gov/water/IPDES.

The CGP can be found on the Idaho Department of Environmental Quality (IDEQ) website at: https://www2.deq.idaho.gov/admin/LEIA/api/document/download/16509. The SWPPP shall have been prepared and submitted to ACHD for acceptance prior to the filing of the NOI. Prior to filing the NOT, the conditions listed in Part 8.2 (Conditions for Terminating CGP Coverage) of the CGP shall be met.

Once a SWPPP has been prepared, the Contractor and ACHD shall both submit an electronic NOI on the website listed above. There is a fourteen-calendar day wait after the acknowledgement of receipt has been posted on the IDEQ website for the SWPPP to be considered approved and construction allowed to commence.

Prior to starting construction, the ACHD accepted SWPPP/CSDC Plan must be implemented. No Construction Activity or Land Disturbing Activity will be allowed to commence until the Contractor has fully implemented the accepted SWPPP/CSDC Plan as required by the District and set forth in the ACHD Policy Manual.

Additionally, the contractor is responsible for installing, maintaining, and removing all Best Management Practices (BMPs) and for all documentation required to keep the SWPPP current.  For compliance with the District’s CSDC Program, the SWPPP/CSDC Plan should address all potential pollutants outlined in the ACHD Policy Manual.

A Rainfall Erosivity Waiver (Waiver) is available and defined in Appendix B Part B.1 of the CGP. If the Waiver is utilized the Waiver shall have been prepared and submitted to ACHD for acceptance prior to the filing of the Waiver through the IPDES E-Permitting website.   If the conditions on which the Waiver is based change, the Contractor is responsible for updating the Waiver and/or development, including filing for NOI and NOT, and implementation of a SWPPP.

BMPs for controlling pollutant transport from the construction site can be found in a few publications including, but not limited to:

- Idaho Department of Environmental Quality, Idaho Catalog of Storm Water Best Management Practices at: https://www.deq.idaho.gov/water-quality/wastewater/storm-water/  or Phone: (208) 373-0502

- United States Environmental Protection Agency – Region 10, Resources and Tools at: https://www.epa.gov/npdes/construction-general-permit-resources-tools-and-templates , or phone: (800) 424-4372

- Idaho Transportation Department, Best Management Practices (BMP) Manual at: https://itd.idaho.gov/env/?target=resources, phone: (208) 334-8476

Measurement and Payment: Payment for work items to implement the SWPPP or CSDC shall be per other specific bid items noted in this contract.

Payment for this plan preparation and approval item will be made under:

SSP 06013 – Stormwater Management Plan Preparation & Implementation …………..…Per Lump Sum

- SSP 08125 – ASPHALT REPAIR - OTHER

Description: This item shall include all work and costs associated with the repair of existing local roads, asphalt driveways, parking lots, and sidewalks abutting the project to match the grade of curbs, sidewalks, driveway approaches, and existing asphalt.

Materials: This item shall include excavation, labor, equipment, and materials necessary to complete placement of a 4” (inch) thickness of 3/4” (inch) aggregate base course, on a compacted sub-grade, and a 3” (inch) thickness of ½” SP-3 Plant Mix Asphalt in accordance with Section 814 of the ACHD supplement to the ISPWC. The asphalt cement performance grade shall be PG 64-28 and shall contain ½% of heat-stable anti-stripping agent per ton of asphalt cement added immediately prior to use at the location of the asphalt batch plant.  Asphalt tack material shall be an SS-1 emulsified asphalt diluted as specified in accordance with ISPWC Division 800 – “Aggregate and Asphalt.”

Workmanship: This item shall also include all costs associated with the furnishing and placement of an asphalt tack coat on the lip of the gutter and on the edges of previously placed asphalt.

Measurement and Payment: Material costs associated with the furnishing and placement of an asphalt tack coat on the lip of the gutter and on the edges of previously placed asphalt are considered incidental to this item.  All placed asphalt paid under this item shall be measured by the square yard.

SSP 08125 – Asphalt Repair - Other	Per Square Yard

- SP 11551– PEDESTRIAN TEMPORARY TRAFFIC CONTROL PLAN

Description: This work shall consist of preparation and implementation of a Pedestrian Temporary Traffic Control (TTC) Plan as needed in the construction area inclusive to provide temporary facilities, including reasonably safe pedestrian routes around the Contractor’s work in accordance with the Americans with Disabilities Act (ADA).

Materials: All materials (signs, barricades, temporary facilities and appurtenances) shall be in accordance with the Americans with Disabilities Act of 1990 (ADA) and Manual on Uniform Traffic Control Devices (MUTCD).

Workmanship: The Contractor shall develop a Pedestrian TTC Plan for work at the construction site based upon his planned construction operation. The Contractor shall submit a written outline of the TTC plan along with a layout of all temporary facilities, barricades and signing for review and approval prior to construction. The TTC plan shall provide the following:

- The needs and control of pedestrians and bicyclists within the roadway and/or public right-of-way, including persons with disabilities in accordance with the Americans with Disabilities Act of 1990 (ADA), Title II, Paragraph 35.130) through a temporary traffic control “TTC” zone shall be an essential part of highway construction, utility work, maintenance operations, and the management of traffic incidents. The primary function of TTC is to provide for the reasonably safe and efficient movement of road users through or around TTC zones while reasonably protecting workers, responders to traffic incidents, and equipment.

- Temporary facilities, including reasonably safe pedestrian routes around work sites, are also covered by the accessibility requirements of the Americans with Disabilities Act of 1990 (ADA) (Public Law 101-336, 104 Stat.327, July 26, 1990. 42 USC 12101-12213 (as amended)). Implementation of TTC plans and installation and maintenance of devices shall be the responsibility of the Contractor performing the construction, alteration and/or maintenance of the roadway or public right-of-way. When an existing continuous sidewalk or street crossing route cannot be maintained for pedestrians because of construction, either temporary walkways with curb ramps are to be provided, or the construction shall be phased to maintain access to the affected addresses. Contractors shall be allowed flexibility as long as the requirements are met.

- The location of the construction project and whether or not accessible facilities are present shall also determine the extent of the needed temporary facilities. The Contractor is only required to maintain practical continuity where accessible facilities already exist. On low speed rural roads that do not have sidewalks and are used by bicyclists, no additional measures are needed as the bicycles can share the available travel lanes with other traffic. On moderate to higher speed rural roads, if a bike lane exists then it should be properly detoured, complete with signage, to provide a safe route through or around the work area. If a road or bridge project affects vehicular traffic to a business, residence, school or any other type of pedestrian generating location with existing accessible facilities, then pedestrian and handicapped access must be maintained.

- A continuous route for all pedestrians, including the disabled and bicyclists, shall be maintained at all times. When existing pedestrian facilities are disrupted, closed, or relocated in a TTC zone, the temporary facilities shall be detectable and include accessibility features consistent with the features present in the existing pedestrian facility. The temporary route should enable pedestrians to bypass the construction site while minimizing the retracing of their steps or going significantly out of their way. Additional consideration must be given to the disabled since they may not have the physical or cognitive ability to improvise (e.g. balancing along the curb or a very narrow path) or use unofficial alternatives (e.g. using an adjacent grass surface). Temporary routes must meet the accessibility guidelines of the ADA for permanent facilities and shall be marked with the proper signage. Should existing crosswalks at signalized intersections be closed or made inaccessible, temporary crosswalks should be painted in an accessible location. Temporary signals should include pedestrian phases.

- Contractors shall not block temporary walkways with Contractor parking, materials piles, signs, rubble or rubbish. Construction equipment and equipment operation must be separated from the temporary walkways. At work zones where higher volumes of pedestrian traffic or school children exist, pedestrian fences or other protective barriers may be needed to prevent access into the construction area.

- Detour and diversion routes, when used for pedestrians and bicyclists, should be evaluated for the following items:

- Direct conflicts between pedestrians and vehicular traffic, work vehicles, and other work activities must be reduced with protective barriers or continuous high contrast fencing (min 36” high with a 6” high toe board). See Manual on Uniform Traffic Control Devices (MUTCD) 6F.68 and 6D.02

- Temporary pedestrian facilities should provide safe, accessible routes that replicate as nearly as practical the most desirable characteristics of the existing facility, and parallel the disrupted route whenever possible. A smooth, continuous hard surface should be provided throughout the entire length of the temporary pedestrian facility. There should be no curbs or abrupt changes in grade or terrain that could cause tripping or be a barrier to wheelchair use. The geometry and alignment of the facility should meet the applicable requirements of the Americans with Disabilities Act Accessibility Guidelines (ADAAG) for Buildings and Facilities. See MUTCD 6D.01 and 6D.02

- Advance information placed at appropriate distances before the work zone allowing pedestrians to make timely decisions about routes through or around the work zone. See MUTCD 6F.14

- Transition information allowing pedestrians to find a safe path through and around work zones, which is critical when the pathway is restricted, diverted or detoured. See MUTCD 6F.14

- Work area information assisting in safe passage of pedestrians through the work zone. This information is needed on all pedestrian routes except detours. See MUTCD 6F.14

- Exit information directing pedestrians back to the original route. See MUTCD 6F.14

- Crosswalk placement at intersections may need additional signage, temporary striping, traffic signal modification, pedestrian signals with audible alarms if justified, proper push button height, and ramps. See MUTCD 6H.29, 6F.80, and 4E.06

- Accommodations for other transit forms (busses, trains etc.) are made. See MUTCD 6D.02

- Requirements of the ADAAG and MUTCD are adhered to.

- Access is maintained to the affected businesses and residences.

- Frequent checks of the pedestrian and bicycle accommodations are made during construction to ensure that the temporary traffic control plan is followed, traffic control devices are maintained in good condition, and safe, accessible pedestrian and bicycle routes are available at all times.

Upon approval the Contractor shall implement the prepared TTC plan. The Contractor shall be responsible for all work associated with the TTC plan including all initial placement, maintenance, relocations, and removal of all signs, barriers and temporary facilities.

Measurement and Payment:  Traffic control devices used for the pedestrian or bicycle detour will be paid under their respective bid items when applicable. Relocation of each device to another location shall be paid under Traffic Control Maintenance.  This item includes all design, submit, modify, inspect, and manage the Pedestrian Temporary Traffic Control Plan. Additional materials, labor, and equipment needed to complete and maintain the approved pedestrian detour will be measured and paid for by Force Account

Payment for this item will be made under:

SP 11551 - Pedestrian Temporary Traffic Control Plan ………………………………Contingency Amount

- SSP 20009 – REMOVE AND RESET FENCE

Description: This item shall consist of all labor, equipment, braces, posts, gates, and all additional appurtenances necessary to remove and reset fence where indicated on the plans, to the existing parallel property line, or as directed.

Materials: Fencing materials shall consist of new materials for fence replacement, which shall match as closely as possible with existing arrangements and shall meet the requirements outlined in the contract documents.

Workmanship:  The Contractor shall ensure the new fence meets installation requirements associated with each type of specified fencing to be removed and relocated.  It shall be the Contractor’s responsibility to coordinate with the property owner to ensure the fence to be removed and replaced is consistent with the compensation summary as approved by the ACHD right-of-way Agent.  If this information is not provided in the contract documents it can be obtained by Contacting Dave Serdar, ACHD right-of-way Supervisor, at 208-387-6224. The fence will be complete and sturdy and placed at the lines and grades indicated on the project plans, or as direct, upon final inspection.

Measurement and Payment:  Removal and Replacement of fencing shall be measured on the per foot basis, complete, in place.  This item and includes any associated cost for permits necessary for approval by the city located within the boundary of the project limit.  This item includes all costs associated with coordination with the property owner to ensure permit compliance and adherence to that agreed upon in the compensation summary.

Payment for this item will be made under:

SSP 20009 – Remove and Reset Fence…………………………………………………Per Lineal Foot

- SP 25072 – FIELD OFFICE

Description: This item consists of furnishing and maintaining a field office and a project meeting

place for the exclusive use of ACHD, at an approved location on or near the project site,

including electrical power, potable water, rental restroom facility with adequate sanitary facilities

and weekly cleaning.

Materials and Workmanship: The field office structure shall consist of a mobile trailer, shall be

weather tight, have adequate lighting and shall have acceptable locking windows and doors. The

Contractor shall secure the necessary hardware to allow ACHD to secure the field office with

locks on all exterior doors and windows.

The field office shall have a minimum of 200 square feet of total area, adequate in ceiling height,

and divided into two separate rooms. One room shall be adequate as a conference area and

conference table for the weekly construction meetings. Each room shall have a minimum of two,

2- outlet receptacles and there shall be at least one external waterproof 2- outlet electrical

receptacle and 115v.

General office requirements include:

Heating and Cooling:        Adequate to maintain an ambient air temperature of 70 degrees

Drafting Table or Desk:    Minimum 3' x 5' in size

Conference Table:            Two tables, minimum 3' x 8' in size, with ten folding chairs

First Aid Kit:                       One with standard supplies, including eyewash

Fire Extinguisher:             One 5- lb min. size, multi-purpose (A, B, C), non-toxic, UL listed 2A: 10B: 10C

Broom and Dustpan:       One of each

Utilities:                             Electrical power and propane or natural gas, depending upon the

furnace heater requirements

Measurement and Payment: The Field Office will be measured by the Lump Sum and shall

include all labor, equipment and materials necessary for the completion of the bid item and for

duration of construction. Any other items required to complete the initial field office installation,

the office relocation, if needed, and removal of the field office upon project completion will not

be measured separately but shall be considered incidental to this item.

Payment for this item will be made under:

SP 25072 – Field Office ……………………………………………………………Per Lump Sum

- SSP 29060 – HYDROSEEDING

Description: This item shall include all work and costs associated with hydroseeding in the areas designated on the plans or as directed by ACHD.

Materials: Hydroseeding shall consist of furnishing and installing, seed, fertilizer, mulch, and water using the hydroseeding method. Seed shall be a dry land grass mixture prepared by a local nursery appropriate for the Treasure Valley. Application rate of the seed mixture shall be 30 lbs/acre.

Each variety of seed shall be tested seed from the latest crop available, and shall be delivered in standard sealed containers labeled in accordance with State and Federal Laws. The label shall show the variety of seed, the percentage of germination, purity and weed content. All varieties of seed shall have a minimum tested germination of 85% and contain a minimum of 80% pure seed by weight. Seed shall not be agitated in the hydro-seeder over 30 minutes.

Fertilizer shall be of any standard brand suitable for use with the hydroseeding method, furnished in moisture proof bags. Each bag shall be marked with the weight and manufacturer's analysis of the ingredients. Fertilizer shall contain a minimum of 22% available nitrogen. Fertilizer shall be applied uniformly at 440 pounds per acre. Fertilizer shall not be mixed with the seed in the hydro-seeder.

Mulch shall be a wood fiber mulch commonly used in the hydroseeding process. Mulch shall be applied at a rate of 2,000 lbs/acre.

Contractor shall provide certification for hydraulically applied erosion control products from the manufacturer that the materials are nontoxic to animals, soil microorganisms, aquatic and plant life, and will not interfere with or impede seed germination or vegetative growth and establishment.

Workmanship: Seeding shall be performed only at times when local weather conditions are favorable for growth, which normally will occur between September 15 and November 30, or between February 15 and May 15. The Contractor shall be responsible to protect and maintain the seeded areas until germination, including watering if necessary.

Measurement and Payment: Construction limits for this item shall be as shown on the plans. Any hydroseeding restoration required beyond the specified construction limits shall be made by the Contractor at his expense and no separate payment will be made, unless additional areas are as directed by the Engineer.

Hydroseeding will be measured per square yard and shall include all labor, equipment and material necessary for the completion of the bid item.  The accepted quantity of Hydroseeding will be paid at the contract unit price for the item listed below.

ITEM 29, “VARIATIONS IN QUANTITIES,” ON PAGE GC16 OF THE ACHD GENERAL CONDITIONS, SECOND PARAGRAPH, shall not apply to this bid item.

Payment for this item will be made under:

SSP 29060 – Hydroseeding	Per Square Yard

- SSP 29101 – REMOVE & RESET SPRINKLER SYSTEM

Description: This item consists of furnishing all labor, equipment and material necessary to remove existing sprinkler systems, install and maintain temporary sprinkler systems during construction, adjust/relocate existing sprinkler systems, or install new sprinkler systems at the locations shown on the plans or as directed by the Engineer.

Materials: All materials shall conform to the ISPWC and all ACHD ADOPTED SUPPLEMENTS and shall be equal to, or of better quality than, existing materials.

Workmanship: Prior to commencement of construction, the Contractor shall document the locations of existing sprinkler systems within the construction zone. Documentation shall include, but is not limited to, type and location of existing sprinkler heads, pipe, controllers, valves and control wires. Documentation shall be provided to the Engineer prior to demolition of existing sprinkler systems. Costs associated with providing documentation of existing sprinkler systems shall be considered incidental to this item.

Adjusted/relocated sprinklers shall be installed to restore adequate coverage to remaining landscape areas and new sod areas. Over-spray onto the roadway and sidewalks will not be allowed.

The Contractor shall maintain all sprinkler systems outside of the construction zone that are impacted by the Contractor’s activities. This may require the Contractor to install temporary sprinkler main lines around the construction zone. All costs associated with installing and maintaining temporary sprinkler systems and providing temporary water during construction shall be considered incidental to this item. Contractor shall cut and cap existing lines and supplement existing systems with additional materials as necessary.

Measurement and Payment:  Remove and Reset Sprinkler System shall be measured by the linear foot of mainline pipe that is adjusted/relocated or newly installed, and shall include all labor, equipment and material as necessary for completion of the bid item.  Providing and adjusting of individual sprinkler heads shall be measured as ten linear feet each.

ITEM 29, “VARIATIONS IN QUANTITIES,” ON PAGE GC16 OF THE ACHD GENERAL CONDITIONS, SECOND PARAGRAPH, shall not apply to this bid item.

Payment for this item will be made under:

SSP 29101 – Remove & Reset Sprinkler System	Per Linear Foot